Wireless Headphone “WL”

For use with MP3 or portable audio device:

1. Insert the audio cable (yellow) into the audio socket of the transmitter, and the other end into the Headphone Out jack on your audio device.

2. Move the function switch to “WL” for Wireless Headphone and the power indicator of transmitter will go on.

3. Turn on the volume/power switch on headphone and adjust the volume, the power indicator on the headphone will go on.

4. Press RESET key, then press and hold SCAN key for 1 second, wait for 5-8 seconds, the sound should send from the transmitter. If you do not hear, continue to press SCAN key slowly until you hear sound from your audio source. If not, repeat step 4 again.

For use with Television:

1. Insert the audio cable (yellow) into the audio socket of the transmitter; connect the ends into the RCA cable, then insert the RCA cable into the RCA AUDIO OUT of the television. When in use with television, it is advisable to mute the television for better audio experience.

2. Move the function switch to “WL” for Wireless Headphone and the power indicator of transmitter will go on.

3. Turn on the volume/power switch on headphone and adjust the volume, the power indicator on the headphone will go on.

4. Press RESET key, then press and hold SCAN key for 1 second, wait for 5-8 seconds, the sound should send from the transmitter. If you do not hear, continue to press SCAN key slowly until you hear sound from your audio source. If not, repeat step 4 again.

Note: For better audio reception, please make sure the 3.5mm cable is not tied or tangled as the cable acts as an antenna during use.

Wireless Internet Audio Chat “CHAT”

1. Connect the transmitter audio cable (yellow) to the audio socket of the transmitter and the other end into the Headphone/Audio Out jack of the PC. Then insert the 2nd audio cable(yellow) into the Microphone socket of the transmitter and the other end to the Microphone Out jack of the PC.

2. Move the function switch to “NET” for Wireless internet audio chat function and the power indicator of transmitter will go on.

3. Turn on the volume/power switch on headphone and adjust the volume, the power indicator on the headphone will go on.

4. Press RESET key and press SCAN key repeatedly until you hear the sound from your PC.

Wireless Monitoring “MNT”

1. Place the transmitter near the person you want to monitor and aim the microphone towards the person.

2. Move the function switch to “MNT” for Wireless monitoring and the power indicator of transmitter will go on. Turn on the volume/power switch on headphone and adjust the volume, the power indicator on the headphone will go on.

3. Press RESET key and press SCAN key repeatedly until you hear the sound from the person to monitor.